Survey Report

of the

City & County of San Francisco

2013 High Precision Network Survey

Prepared by

McGee Surveying Consulting and F3 & Associates, Inc.

February 28, 2014

INDEX

Page          	Subject

2          PROJECT OVERVIEW

3          PROJECT DATUMS, REFERENCE SYSTEM

4          GEOMETRIC COORDINATES OF CONTROLLING STATIONS (CORS & CGPS)

### 4          NETWORK DESCRIPTION

6          ADJUSTMENTS & ANALYSIS: IGS08(2005) Epoch 2013.54, NAD83(2011) Epoch 2013.54,

NAD83(2011) Epoch 2010.00 and Geoid Model Analysis

### 10        DATA COLLECTION, PROCESSING and EQUIPMENT

11        ACCURACY: LOCAL & NETWORK

13        TRANSFORMATIONS: 1999 NAD83 (1991) SPCS to NAD83 (2011) 2010.00 SPCS

1999 NAD83 (1991) SPCS to NAD83 (2011) 2010.00 CCSF-CS

14        RECOMMENDATIONS

APPENDIX

15        Glossary

16        Geodetic Coordinate List: NAD83(2011) and IGS08(2005)

17        Plane Coordinate List: NAD83 SPCS & CCSF Coordinate System (Low Distortion Projection)

18        Map: CCSF 2013 Regional GNSS Network

19        Map: CCSF 2013 HPN GNSS Network

20        CORS Reference Data: Coordinates, HTDP Solutions, NGS Data Sheets and Short-Term Time Series

Survey Report

of the

City & County of San Francisco

2013 High Precision Network Survey

Prepared by

McGee Surveying Consulting and F3 & Associates, Inc.

February 28, 2014

### OVERVIEW

Project:  City and County of San Francisco High Precision Network Survey (CCSF-HPN)

Surveyed by:  City and County of San Francisco supported by McGee Surveying Consulting, F3 & Associates, Inc. and Frederick T. Seher & Associates, Inc.

This document serves as a summary report of the City and County of San Francisco (CCSF) 2013 “B” Order Control Survey. A Record of Survey will follow incorporating the content of this Report and containing detailed descriptions of the points. This survey established 20 permanent high precision control points in July 2013 referred to as the CCSF-2013 HPN. This network provides the framework for establishing a densification network to support general survey activities, GIS, and for measuring annual and episodic earth movements. The field surveys were planned and coordinated in a joint effort by CCSF, Michael McGee, PLS3945 and F3 & Associates, Inc. Michael McGee, PLS was responsible for the final processing of the observations, network adjustments, analysis and reports. The Global Navigation Satellite System (GNSS) was used to determine positions based on the North American Datum of 1983 (NAD83) and International GNSS Service 2008 Reference Frame (IGS08). The survey is referenced to four National Geodetic Survey (NGS) Continuously Operated Reference Stations (CORS) in the region.

CCSF Regional & High Precision Network

PROJECT DATUMS, REFERENCE SYSTEM

Geometric Datums: North American Datum of 1983 - NAD83(2011) Epoch 2010.00 realization and IGS08(2005) Epoch 2013.54 (July 17, 2013 average date of field survey)

Reference Network:  The survey is referenced to four NGS CORS stations (see NGS Data Sheets in Appendix)

Vertical Datum: CCSF 2013 NAVD88 Vertical Datum. Note, this survey evaluated the use of GNSS technology for establishing accurate orthometric heights and modeled the orthometric height of UCSF.

Reference Network: CCSF 2013 Second Order Leveling Network

Projections: The plane coordinates published in the survey are in units of meters and feet, and in two projections. Coordinates are provided in the California State Plane Coordinate System (SPCS) Zone III and in a local custom coordinate system created by this survey and referred to as the City & County of San Francisco Coordinate System (CCSF-CS).

The CCSF-CS is a low distortion grid projection designed for CCSF to be a ground coordinate system. CCSF varies from sea level to approximately 1000 feet in elevation. To minimize the differences in ground and grid distances, the projection surface was positioned at the most common ground height in the County taken at an ellipsoid height of 44.50 meters (146.0 feet). The average geoid height is -32.6 meters (-107 feet) and the NAVD88 height of the projection surface is 77 meters (253 feet). At this height the combined scale factor is 1.0 and the distortion is zero. Changes in height will increase or decrease the scale 4.8 ppm for every 30.5 meters (100 foot). This coordinate system provides a grid scale distortion of less than 1:100,000 (10 ppm) in most parts of CCSF except for the higher elevations. At the Central Meridian, North coincides with Geodetic North referenced to the GRS80 ellipsoid, centered in the NAD83(2011) 2010.00 Epoch reference frame. The Convergence Angle varies +/- two minutes as shown in the Appendix in the Plane Coordinate List. The projection specifications (similar to an SPCS projection) for input in user’s software follow.

Projection: Transverse Mercator, Ellipsoid: GRS-80, Scale: 1.000007, Latitude of Origin: 37º45’00”,

Central Meridian: -122º27’00”, False Northing: 24000.0 meters, False Easting: 48000.0 meters.

The average Scale Factor, Height Reduction Factor and Convergence Angles listed below for the 20 HPN points provide a general comparison of the two projections.

State Plane Coordinates Zone 3:

Combined Factor= 0.99992496 Convergence Angle= -1-11-34

San Francisco Coordinate System:

Combined Factor= 1.00000275

Convergence Angle= +0-00-04

The specific values for each point are listed in the Appendix. Applying the average combined factors to a ground distance of 1000 foot equals 1000.003 feet in the CCSF-CS whereas in the SPCS it is 999.925 feet.

Historical CCSF Control Network:

Figure 1: 1999 Network Points and 2013 HPN Points

A map on file with the CCSF titled “City and County of San Francisco Precise Horizontal Survey Control” dated May 2001 depicts a GPS Survey based on NAD83 (1991) 1991.35 Epoch with State Plane Coordinates published in 1991, 1998 and 1999. The 1999 Network (red) and 2013 HPN

(green) are shown in Figure 1. This 2013 survey supersedes the 1991-1998-1999 surveys.

Reference Network Velocity:  The North American Datum of 1983 is referenced to the North American Plate. CCSF sits on the Pacific Plate and is situated between the San Andreas Fault approximately 2 kilometers west of the southwest corner of CCSF and the Hayward Fault approximately 15 kilometers east of the northeast corner of CCSF. CCSF is moving 3.1 centimeters (0.10 feet) north and 2.1 centimeters (0.07 feet) west (N34°W 0.12 feet) per year relative to the North American Plate at the CGPS station UCSF as predicted by the NGS Horizontal Time Dependent Positioning (HTDP) model. Continued monitoring of the 2013 CCSF-HPN over time will develop a model of differential movements across the County.

GEOMETRIC COORDINATES OF THE CONTROLLING CORS & CGPS STATIONS

The following positions were constrained or derived in the Adjustments discussed hereafter.

NAD83(2011) Epoch 2010.00 Positions per NGS Data Sheets (see Appendix)

ID     Latitude(dms)    W.Longitude(dms)   EH(mtrs)_ NGS PID        NAME_____

P176   37-28-18.36834    122-21-25.65069    434.339   DN7544  MILLSCREEKCN2007 GRP

P224   37-51-50.01427    122-13-08.56363    407.873   DH3881  SIBLEYVOLCCN2005 GRP

TIBB   37-53-27.13938    122-26-51.31741    -20.565   AI4507  TIBURON PENINSUL GRP

WINT   37-39-09.50579    122-08-25.99416    -28.259   AI4510  WINT_BARD_CN1991 GRP

NAD83(2011) Epoch 2013.54** per NAD83 velocities applied to Epoch 2010.00 Positions

ID     Latitude(dms)    W.Longitude(dms)   EH(mtrs)       Velocities (mm)____

P176   37-28-18.37230    122-21-25.65398    434.334   N 34.5  E-22.9  Up -1.3*

P224   37-51-50.01654    122-13-08.56563    407.869   N 19.8  E-13.8  Up -1.2

TIBB   37-53-27.14260    122-26-51.31998    -20.570   N 28.1  E-17.7  Up -1.4*

WINT   37-39-09.50872    122-08-25.99666    -28.264   N 25.5  E-17.3  Up -1.3*

NAD83(2011) Epoch 2013.54 Results of the Constrained Adjustment to the CORS positions listed above (see Adjustments & Analysis)

CCSF   37-44-55.64324    122-24-01.58597    -15.950

EBMD   37-48-54.01939    122-17-01.65016    -15.403

MHDL   37-50-32.35159    122-29-39.55028     66.403

P178   37-32-04.25894    122-19-56.46131    129.572

UCSF   37-45-46.67553    122-27-29.29178    155.123

IGS08(2005) Epoch 2013.54 per IGS08 velocities applied to Epoch 2005 Positions

ID     Latitude(dms)    W.Longitude(dms)   EH(mtrs)       Velocities (mm)___

P176   37-28-18.38315    122-21-25.71047    433.810   N 21.3  E-36.2  Up  0.0*

P224   37-51-50.02759    122-13-08.62234    407.349   N  6.8  E-27.5  Up  0.1

TIBB   37-53-27.15354    122-26-51.37682    -21.084   N 14.8  E-31.4  Up  0.0*

WINT   37-39-09.51974    122-08-26.05320    -28.790   N 12.5  E-31.0  Up  0.0*

*HTDP v3.2.3 velocities applied to CORS with less than 2.5 years of data

**Epoch of Survey

NETWORK DESCRIPTION

The CCSF GNSS Survey is comprised of a bay area Regional Network shown in Figure 2 and the CCSF High Precision Network shown in Figure 3. The Regional Network utilized four of the nearest operating NGS CORS stations (TIBB, P224, WINT and P176) as a basis for recovering IGS08(2005) Epoch 2013.54 and NAD83(2011) Epoch 2010.00 reference frames. Four CGPS stations (EBMD, MHDL, P178 and UCSF) were included to add strength and redundancy to the Regional Network. A private RTN station (CCSF) was included to establish a position relative to the CCSF-HPN. Three vectors or baselines were computed for each connection shown in Figure 2 using 24 hour observations staggered every other day during the field campaign.

The 2013 CCSF-HPN (shown in Figure 3 and the Appendix) consists of 20 primary high precision points distributed across CCSF. The field campaign took place during the week of July 15-19, 2013 (average epoch 2013.54). Four crews operated four Leica GS15 GNSS receivers mounted on fixed-height poles. The receivers and fixed height poles were calibrated, and a validation survey of equipment and procedures took place prior to the field campaign.

Figure 2: Regional Network

The field campaign began on July 15 (Day1) with a base receiver occupying Point #101 (Corona Heights) for the day while the three crews occupied all points for 45 minutes each developing a radial network. On Day 2, a second radial network was developed with a base receiver occupying Point #102 (Marrietta Drive) as a base station while three crews occupied points for 45 minutes completing a second independent occupation of all points. On Day 3 and 4, a tandem operation was conducted with four crews working in unison collecting 45

minutes in common at their assigned points to complete the inner network connections between all adjacent points. At the end of the field campaign, all HPN points were occupied four or more times on different days and under different constellations.

Figure 3: City & County 2013 High Precision Network

In addition to the 20 primary HPN control points, three secondary points 201 (Tidal), 202 (Sloat) and 203 (Army) shown on the map of the 1999 GPS Survey on file with the CCSF Public Works referred to above were occupied twice with a fifth receiver for 15-30 minutes. These points are not part of the HPN. These points were included to determine the relationship between the 1999 CCSF reference frame and the 2013 NAD83(2011) 2010.00 Epoch reference frame established by this survey. Transformations from the 1999 reference frame to this 2013 survey are provided hereafter.

ADJUSTMENTS & ANALYSIS

Non-trivial vectors were processed from the observations and evaluated in the network adjustments listed below. The Regional Network connected nine stations including four CORS, four CGPS stations and one private continuously operated RTN station. The Regional Network contains 57 vectors averaging 20 kilometers in length with a maximum of 38 kilometers. The two-dimensional residuals averaged 0.002 meters with a standard deviation of 0.002 meters and a maximum of 0.010 meters. The absolute value of the vertical residuals averaged 0.002 meters with a standard deviation of 0.002 meters and a range of -0.007 to +0.008 meters. The 2013 CCSF-HPN connected 20 points, three secondary points, two CGPS stations and the private RTN station. The network contains 83 vectors averaging 4.3 kilometers in length with a maximum of 8.3 kilometers. The two-dimensional residuals averaged 0.003 meters with a standard deviation of 0.002 meters and a maximum of 0.010 meters. The absolute value of the vertical residuals averaged 0.003 meters with a standard deviation of 0.003 meters and a range of -0.009 to +0.016 meters.

Six network adjustments were processed to develop the geodetic and plane coordinates in two reference frames at different epochs. An overview of the adjustments follow with details thereafter.

Adjustment #1 and #2: Developed IGS08(2005) Epoch 2013.54 positions for referencing future secular and episodic movements of the region and CCSF. The IGS08(2005) positions of the four CORS were obtained from the NGS and moved to Epoch 2013.54 (epoch of this survey) using the NGS HTDP v3.2.3 velocity model. Adjustment #1 fixed WINT to evaluate the vector residuals and closures on the remaining three CORS. Adjustment #2 constrained the network to all four CORS to develop IGS08(2005) Epoch 2013.54 positions.

Adjustment #3 and #4: Developed NAD83(2011) Epoch 2013.54 positions for referencing the Regional Network. The NAD83(2011) Epoch 2010.00 positions of the four CORS were obtained from the NGS and moved to Epoch 2013.54 (epoch of this survey) using the HTDP v3.2.3 velocity model.

Adjustment #3 fixed WINT to evaluate the closures on the remaining three CORS.

Adjustment #4 constrained the network to all four CORS to develop NAD83(2011) Epoch 2013.54 positions on the four CGPS stations.

Adjustment #5: Developed NAD83(2011) Epoch2010.00 positions for CCSF-HPN. The NAD83(2011) Epoch 2013.54 positions of stations UCSF, MHDL and CCSF, determined in Adjustment #4, were moved to Epoch 2010.00 using the HTDP v3.2.3 velocity model.

Adjustment #5 fixed the CGPS station UCSF at the Epoch 2010.00 position to evaluate the closures on MHDL and CCSF, and to develop positions on the 2013 CCSF-HPN.

Adjustment #6: Analyzed the Geoid 2012A Model utilizing a seven parameter conformal transformation with the scale parameter fixed at 1.0 to best fit the GNSS measurements to the NAVD88 Heights of the HPN points determined in the CCSF 2013 Second Order Leveling Network Survey (Report on file with CCSF). The adjustment demonstrates the application and expected results of using GNSS survey technology to develop orthometric heights in CCSF.

IGS08(2005) Epoch 2013.54 Adjustments

##### Adjustment 1:  3D Minimally Constrained Adjustment

The four nearest operating CORS form the basis for recovery of the IGS08 reference frame for this survey. IGS08(2005) Epoch positions and velocities were obtained from the NGS website for the CORS. The HTDP v3.2.3 model was used to update the CORS positions to the 2013.54 Epoch of this survey. Four CGPS stations and a private RTN station CCSF were included in the adjustments.

Station WINT was fixed in a Minimally Constrained Adjustment to determine preliminary latitude, longitude, ellipsoid heights at other stations and compute closures. The results follow with the coordinate differences (closures) from the IGS08 positions to the computed positions listed in meters below. The position for UCSF was determined from the mean of three 24 hours observations submitted to OPUS and used as a check on the results of this adjustment as shown.

Coordinate Differences: IGS08 to Computed

Station   dN       dE       dZ_

P176    0.007   -0.004    0.006

P224    0.012    0.003    0.002

TIBB    0.003   -0.001   -0.015

WINT    0.000    0.000    0.000  Fixed

UCSF    0.004    0.000   -0.018  OPUS IGS08 position to computed position

Note:  The differences between the published and computed positions range 0.000 to 0.012 meters in north,

-0.001 to +0.003 meters in east and -0.015 to +0.006 meters in the up direction. Network loop closures were less than a centimeter. The horizontal check on the OPUS position of UCSF is 0.004 meters.

##### Adjustment 2A:  3D Adjustment Solving for Transformation Parameters from WGS84 to IGS08

WGS84 is based on the IGS08 reference frame. In this adjustment the rotations and scale were solved to verify the consistency of the measured network in WGS84 and the IGS08 reference frame as represented by the positions of the CORS. The expectation are zero and the negligible results confirm the consistency of the two frames.

WGS84 to IGS08(2005) Datum Transformation

Scale Factor 1.000000068467 = -0.068 PPM (1:14,700,000)

Rotation Around North Axis =  -0.071 Sec

Rotation Around East Axis =   -0.055 Sec

Rotation Around Vert. Axis =  -0.015 Sec

##### Adjustment 2B:  3D Constrained Adjustment

All four CORS were constrained for latitude, longitude and ellipsoid heights to develop IGS08(2005) 2013.54 Epoch positions on the network. The horizontal difference of 0.001 meters with the UCSF position obtained from the CORS Network utilizing the NGS OPUS Tool confirms the processes used in this survey.

Coordinate Differences: IGS08(2005) 2013.54 Epoch to Computed

Station   dN       dE       dZ_

UCSF   -0.001    0.001   -0.014 OPUS IGS08 position

Note:  This Adjustment includes the CCSF-HPN. The results are listed in the Appendix Coordinate List.

NAD83(2011) Epoch 2013.54 Adjustments

##### Adjustment 3:  3D Minimally Constrained Adjustment

The four nearest operating CORS form the basis for recovery of the NAD83 reference frame for this survey. NAD83(2011) 2010.00 Epoch positions and velocities were obtained from the NGS website for the CORS. The HTDP v3.2.3 model was used to update the CORS to 2013.54 Epoch of this survey for this adjustment. Four CGPS stations and a private RTN station CCSF were included in the adjustment.

CORS station WINT was fixed in a Minimally Constrained Adjustment to determine preliminary latitude, longitude, ellipsoid heights at the three other CORS and compute closures. The results follow with the coordinate differences (closures) in meters from the NAD83(2011) 2013.54 Epoch to the computed positions listed below. The position for UCSF was determined by the SOPAC SECTOR utility and used as a check on the results of this adjustment as shown below.

Coordinate Differences: NAD83(2011) 2013.54 Epoch to Computed

Station    dN       dE       dZ_

P176     0.006   -0.004    0.005

P224     0.012    0.004    0.006

TIBB     0.004   -0.001   -0.010

WINT     0.000    0.000    0.000  Fixed

UCSF     0.009   -0.001   -0.004  Difference from SOPAC SECTOR position to computed

Note: The differences between the published and computed positions range 0.000 to 0.012 meters in north,

-0.001 to +0.004 meters in east and -0.010 to +0.006 meters up direction. The position of UCSF as determined by SECTOR is referenced to the NAD83(2007) Adjustment Epoch 2013.54 whereas this adjustment is referenced to NAD83(2011) Adjustment Epoch 2013.54. The horizontal difference of 0.009 meters is indicative of the similarities of the two national adjustments.

##### Adjustment 4:  3D/Ellipsoid Heights – Constrained Adjustment

All four CORS were constrained for latitude, longitude and ellipsoid heights to develop NAD83(2011) Epoch 2013.54 positions. Notwithstanding the 2007 verses the 2011 Adjustments, the horizontal difference of 0.004 meters at UCSF indicates the compatibility of these results with those obtained from the SECTOR utility.

Coordinate Differences: NAD83(2011) 2013.54 Epoch to Computed

Station    dN       dE       dZ_

UCSF     0.004   -0.001   -0.004 Difference from SOPAC SECTOR position to computed

NAD83(2011) Epoch 2010.00 Adjustments

##### Adjustment 5:  3D Minimally Constrained Adjustment of the CCSF-HPN

The NAD83(2011) 2010.00 Epoch positions at CCSF, MHDL and UCSF were derived by applying HTDP v3.2.3 velocities to the 2013.54 Epoch positions resulting from Adjustment #4. UCSF was fixed in a minimally constrained adjustment to determine latitude, longitude, ellipsoid heights at other stations and compute closures. The results follow with the coordinate differences (closures) in meters from the record positions to the computed positions listed below. The source for the record positions of 107, 201 and 202 are the NGS Data Sheets for the listed PID’s.

Coordinate Differences: Record to Computed

Station    dN       dE       dZ_   NAD83 Epoch        Source

107      -0.034    0.007   -0.040   2010.00     NGS PID AB7679 = HPGND CA04GF

201      -0.029    0.002   -0.045   2010.00     NGS PID AE5209 = N TIDAL

202      -0.053    0.017   -0.031   2010.00     NGS PID AB7677 = HPGND CA04GE

203       0.537   -0.362      -     1991.35     1999 CCSF Control Survey-NAD83(1991)

CCSF     -0.005    0.002    0.007   2010.00     HTDP Applied to Adj#4 Position

MHDL     -0.003    0.003   -0.006   2010.00     HTDP Applied to Adj#4 Position

UCSF     -0.000   -0.000   -0.000   2010.00     HTDP Applied to Adj#4 Position, Fixed

Note: The differences at 203 reflect the difference between the 1991.35 and 2010.00 Epochs. The closures on the HTDP derived positions for CCSF and MHDL range 0.000 to -0.005 meters north, +0.002 to +0.003 meters east and -0.006 to +0.007 meters in the up direction and are at the noise level of the HTDP model; therefore, rather than distort the measured vectors in a constrained adjustment, the results of this adjustment were accepted to establish NAD83(2011) 2010.00 Epoch for the CCSF-HPN. Note: The Adjustment results are listed in the Appendix Coordinate List.

Geoid Model Analysis: Adjustment of Ellipsoid Heights incorporating Geoid Heights to Determine NAVD88 Orthometric Heights:

Two methods for determining NAVD88 Heights in a GPS/GNSS survey are discussed here. The first method approximates NAVD88 Heights by applying geoid heights to the measured ellipsoid heights using the equation H=h-N (Orthometric Height = Ellipsoid Height - Geoid Height). The accuracy of this method when applying the NGS Geoid 2012A to NAD83(2011) 2010.00 Epoch ellipsoid heights in San Francisco is approximately 0.06 meters as demonstrated below.

The second method takes advantage of the relative precision of modeled geoid heights. The Geoid 2012A Model was incorporated in a seven-parameter conformal transformation with the scale parameter fixed at 1.0 to best fit the NAVD88 Heights established on 20 HPN points determined by precise differential leveling (see the “Report on the City & County of San Francisco 2013 Second Order Leveling Network Survey and the 2013 NAVD88 Vertical Datum” on file with the County surveyor).

The NAD83(2011) 2010.00 Epoch Ellipsoid Height differences were combined with the Geoid 2012A Heights shown below in Column E and best fit to the leveled heights at the HPN points shown in Column B. This has the effect of best fitting the Geoid 2012A surface through the NAVD88 Heights at all 20 points. The results shown in Column A are the differences in meters from the leveled heights to the computed heights based on GNSS measurements. This is a best fit (least squares) solution therefore the mean of the differences is zero. The Standard Deviation of the differences is 0.004 meters (0.01’) and the range is -0.007 to +0.007 meters (+/-0.02’). The solved rotations are +0.213 seconds around the north and +0.243 seconds around the east axis. The rotations represent the tilts applied to the Geoid 2012A model to best fit the HPN orthometric heights which represent a geoid surface. The maximum effect of this tilt if unaccounted for is 1.6 mm per kilometer (0.008’ per mile) and is absorbed in a constrained adjustment. The orthometric height of UCSF was modeled in this adjustment.

As an additional check on the differential leveling to the highest HPN points, a constrained adjustment was processed fixing HPN points with orthometric heights less than 8 meters  around the north and east shoreline. As a result, points 101 and 102 were found at -0.005 and +0.001 meters respectively. A one way shortest distance from the north shoreline would allow 0.006 meters for First Order Class I.

Geoid 2012A is a “hybrid” geoid model. Hybrid means it is based on GPS measured ellipsoid heights on a sampling of NAVD88 benchmarks over a large area to estimate the height of the NAD83(2011) zero ellipsoid surface above the NAVD88 zero surface. The measured NAD83(2011) 2010.00 Epoch Ellipsoid Heights (Column C) minus the leveled NAVD88 Heights (Column B) are the measured geoid heights at the HPN points (Column D) determined by this survey. The error in the Geoid 2012A hybrid model in San Francisco is 0.06 meters (Column F) being the difference in the computed Geoid 2012A Heights (Column E) and the measured geoid heights (Column D).

The recommended approach for determining orthometric heights with GNSS is to surround a project with three CCSF-HPN points (four preferred for redundancy) and compute a seven-parameter transformation that includes the Geoid 2012A Model to best fit the HPN 2013 NAVD88 Heights. The accuracy will depend on the combined accuracy of the NAVD88 heights of the HPN points, the GNSS measured ellipsoid height differences, the geoid heights and the effects of the tilt between the geoid model surface and the actual geoid surface. The largest source of error is usually in the measured ellipsoid heights. Following the specifications and procedures used in this survey, an orthometric height accuracy of 0.007 meters (0.02 feet) is obtainable.

As a matter of information, the NGS publishes the US Gravimetric Geoid 2012 (USGG2012) which is a gravimetric model referenced to the GRS80 ellipsoid, centered in the IGS08(2005.00) reference frame, and is the best geo-potential surface that approximates Mean Sea Level (MSL). USGG2012 is applied to WGS84 = IGS08 Ellipsoid Heights to obtain estimated heights referenced to global mean sea level (geoid). Utilizing the USGG2012 Model in a transformation to best fit the NAVD88 heights of the HPN points (described above) returned essentially the same results as the hybrid model; however, the rotations were +0.138 and +0.120 seconds around the north and east axis respectively, representing a negligible improvement. A new North American Vertical Datum is expected to be introduced in less than ten years. The origin or zero height is expected to be the geo-potential surface described above which is approximately 0.9 meters lower than the NAVD88 Datum and approximates the NGVD29 Datum.

### DATA COLLECTION, PROCESSING and EQUIPMENT

Five Leica GS15 geodetic GNSS receivers/antennas listed below were mounted on fixed height poles to collect, and store satellite signal data. The GS15 receivers tracked Navstar GPS and GLONASS satellites. Prior to initiating the field observations a calibration of the fixed height poles was conducted with a theodolite to verify their heights and plumb. The top of the poles were found to be plumb within 1 millimeter of the point. There were no equipment failures during the survey.

Date of Field Surveys: July 15 to July 19, 2013 (Average Date 2013.54)

GNSS Survey Parameters:

Observations: Static occupations collected GPS and GLONASS signal data at a 15 second epoch rate for 45 minutes at all points with 8 hours at the base station setups for the radial observations on the first two days of the field campaign. Observations at the CORS and CGPS stations were GPS only for 24 hour at a 15 second epoch rate and downloaded from the Internet.

Constellation:  32 healthy US Navstar GPS satellites; 24 healthy Russian GLONASS satellites.

Satellite Observations:  12-21 satellites observed with a minimum of 6 GPS and 6 GLONASS at any time;

GDOP < 2; Elevation Mask for Data Collection set at 10° and post processed at 15°.

Observables:  GPS L1 & L2 Carrier wave, GLONASS L1 & L2.

Ephemeris:  Precise GPS and GLONASS for static post processing.

Weather Conditions: Generally overcast marine layer and mild temperatures.

Space Weather:  Boulder K Index = 1-3 (gauges ionospheric activity on a scale of 0-9, <5 preferred).

The K Index was 1-2 on all days except on July 15 it was 3.

Equipment:

GNSS Base Receiver Unit No.:  B, Operator: JTM, RA;

Receiver Make & Model:  Leica GS15; Antenna Make & Model:  Leica GS15

Antenna Mount: Fixed Ht. Pole B; Antenna Height: 1.800m

GNSS Base Receiver Unit No.:  C, Operator: DPL;

Receiver Make & Model:  Leica GS15; Antenna Make & Model:  Leica GS15

Antenna Mount: Fixed Ht. Pole C; Antenna Height: 1.800m

GNSS Base Receiver Unit No.:  D, Operator: GP;

Receiver Make & Model:  Leica GS15; Antenna Make & Model:  Leica GS15

Antenna Mount: Fixed Ht. Pole D; Antenna Height: 1.800m

GNSS Base Receiver Unit No.:  E, Operator: AV;

Receiver Make & Model:  Leica GS15; Antenna Make & Model:  Leica GS15

Antenna Mount: Fixed Ht. Pole E; Antenna Height: 1.800m

GNSS Base Receiver Unit No.:  F, Operator: MM, PC;

Receiver Make & Model:  Leica GS15; Antenna Make & Model:  Leica GS15

Antenna Mount: Fixed Ht. Pole F; Antenna Height: 1.800m

CORS & CGPS National Geodetic Survey Antennas

Note, absolute antenna models were used in the baseline processing and measurements were referenced to the monument not the antenna reference point (ARP).

Stat.   Antenna         Radome

CCSF    AX1202 GG       NONE

EBMD    LEIAR10         NONE

MHDL    ASH700936C_M    SNOW

P176    TRM29659.00     SCIT

P224    TRM29659.00     SCIT
P178    TRM29659.00     SCIT

TIBB    ASH701945B_M    SCIT

UCSF    ASH701945B_M    SCIT

WINT    ASH700936D_M    CAFG <no model available, substituted “ASH700936D_M NONE”

Rinex Data, Antenna Models, Precise Ephemeris and Processing Software: Rinex files for the CORS and CGPS were imported from the NGS and SOPAC. Absolute antenna models were imported from the NGS and the Precise Ephemeris was imported from the NGS for the GPS and IGS for the GLONASS constellations. The Precise Ephemeris was used for all static post-processing in IGS08. The Baseline Processing was performed in Leica Geomatics Office (LGO) v8.1. The Network Adjustments were performed in Starnet v7.2.

ACCURACY: LOCAL & NETWORK

This survey conforms to the requirements of Public Resources Code Section 8801 through 8819 and 8850 through 8880. This survey conforms to the intent of the FGCC “Geometric Geodetic Accuracy Standards and Specifications for Using GPS Positioning Techniques” (1989), the California Geodetic Control Committee (CGCC) “Specifications for High-Production GPS Surveying Techniques” (1993), and NOAA Technical Memorandum NOS NGS 58 "Guidelines for Establishing GPS-Derived Ellipsoid Heights”.

Vector Residuals:  The number of vectors, two-dimensional vector residuals and the absolute value of the vertical residuals resulting from the minimally constrained adjustment are listed below in meters.

Two Dimensional Residuals         Vertical Residuals (absolute values)

No.  Average   Std.Dev. Maximum        Average   Std.Dev.        Range

CCSF HPN         83    0.003     0.002    0.010          0.003     0.003    -0.009 to +0.016

Regional CORS    57    0.002     0.002    0.010          0.002     0.002    -0.007 to +0.008

Local Accuracy:  The local vector lengths, precisions, relative distance accuracy and relative vertical accuracy resulting from the minimally constrained adjustment at the 95% Level of Confidence are listed below in meters.

Lengths           PPM Precisions      Relative Distance Error      Rel.Vert. Error

Network      Vary       Average   Vary     Average    Average Maximum  Precision    Average  Maximum

CCSF HPN     1675-8291     4267   0.4-2.3  1.1 ppm    0.004   0.005    1:1,067,000  0.003    0.004

Regional     5322-37896   20224   0.1-0.6  0.2 ppm    0.003   0.003    1:6,741,000  0.003    0.004

Following the criteria of the former FGCC classification system described in “Geometric Geodetic Accuracy Standards and Specifications for Using GPS Positioning Techniques” (1989), the maximum allowable relative error at the 95% confidence level is defined as s=√(e²+(0.1pd)²) where e is the base error, p is the ppm and d is the distance in kilometers. For the CCSF-HPN, B-Order is equivalent to √(.8²+(0.1·1·4.3)²) = 0.9 centimeters and A-Order is equivalent to √(.5²+(0.1·0.1·4.3)²) = 0.5 centimeters. Loop closures for the HPN vary between 1 ppm and 5 ppm. In conclusion, this survey exceeds a B-Order classification (1:1,000,000) under the former system and the Regional Network exceeds an A-Order classification (1:10,000,000).

Network Accuracy:  The Standard Deviations (68% Level of Confidence) of the latitude, longitude and ellipsoid heights and the 95% Level of Confidence of the Horizontal Positions and Ellipsoid Heights for the Regional Network and the HPN are listed below in meters. This table allows users to calculate the propagated error for future surveys based on the HPN positions.

For P176, TIBB and WINT the NGS Data Sheets state “Formal positional accuracy estimates are not available for this CORS because its coordinates were determined in part using modeled velocities. Approximate one-sigma accuracies for latitude, longitude, and ellipsoid height can be obtained from the short-term time series”. Standard Deviations were only available for P224 (more than 2.5 years of data). Therefore, for consistency the positional accuracy estimates for all CORS stations (P176, P224, TIBB and WINT) were taken from the “Short Term Time Series” available at the NGS website. These one-sigma accuracy estimates were used for weighting the constrained adjustment to develop Network Accuracies.

These geodetic control data meet the 5-millimeter Local Accuracy Standard for the horizontal coordinate values, the 5-millimeter Local Accuracy Standard for the vertical coordinate values (heights), the 1-centimeter Network Accuracy Standard for the horizontal coordinate values, and the 1- centimeter Network Accuracy Standard for the vertical coordinate values (heights) at the 95-percent confidence level according to the reporting standard published by FGDC in "Geospatial Positioning Accuracy Standard, Part 2, Geodetic Control Networks", FGDC-Std-007, 2-1998.

Transformation 1999 NAD83(1991.35 Epoch) SPCS > 2013 NAD83(2011) 2010.00 Epoch SPCS

A map is on file with CCSF titled “City and County of San Francisco Precise Horizontal Survey Control” dated May 2001. The survey is based on NAD83 (1991) 1991.35 Epoch with adjustments published in 1991, 1998 and 1999. The 1999 State Plane Coordinate values shown on Sheets 4 & 5 of 7 for ARMY=203,  CANDLESTICK=107, SLOAT=202 and TIDAL=201 were used to compute a four parameter conformal transformation to convert 1999 State Plane Coordinates in feet to this 2013 survey.

INPUT COORDINATES (Feet)

1999 NAD83(1991)1991.35) SPCS         2013 NAD83(2011)2010.00 SPCS

PT#          North(1)      East(1)         PT#    North(2)     East(2)

CANDLESTICK 2085128.546  6013911.480       107  2085130.260  6013910.280

TIDAL       2121772.462  5993470.060       201  2121774.233  5993468.889

SLOAT       2095678.561  5984226.406       202  2095680.395  5984225.175

ARMY        2100667.364  6012652.104       203  2100669.127  6012650.919

TRANSFORMATION SOLUTION RESIDUALS (Feet)

1999           2013       North    East    N.Azim & Dist

CANDLESTICK    107       -0.019   +0.018     138° 0.026

TIDAL          201       -0.031   -0.009     197° 0.032

SLOAT          202       +0.034   -0.011     342° 0.036

ARMY           203       +0.015   +0.003      11° 0.016

Root Mean Square of the North and East Residuals =    0.02

Scale Factor =  1.00000077      Standard Deviation =  0.00000078

Rotation     = +0° 00' 00.4"    Standard Deviation =  0° 00' 00.2"

TRANSFORMATION EQUATIONS:   N2=A1*N1-A2*E1+A4   E2=A2*N1+A1*E1+A3

A1= 1.0000007745  A2= 0.0000019602  A3= -9.96251  A4= 11.90684

The scale and rotation are negligible as expected between epochs of the same datum. The transformation primarily accounts for the north 1.7± feet and west 1.2± feet shift between the 1991.35 and the 2010.00 Epochs.

Utilize the Transformation Equations and constants listed to convert 1999 NAD83 (1991) Epoch 1991.35 SPCS to NAD83 (2011) Epoch 2010.00 SPCS in CCSF. The horizontal accuracy of the computed positions is estimated at 0.05 feet at the 95% level of confidence based on the residuals.

To convert the 1999 Survey NAD83(1991) 1991.35 Epoch SPCS in feet to NAD83(2011) 2010.00 Epoch of the City & County of San Francisco Coordinate System (CCSF-CS) in feet use the following transformation.

Transformation: 1999 NAD83(1991) SPCS > NAD83(2011) 2010.00 Epoch CCSF-CS

INPUT COORDINATES (Feet)

1999 NAD83(1991)1991.35) SPCS             2010.00 CCSF-CS

PT#         North(1)      East(1)         PT#   North(2)    East(2)

CANDLESTICK 2085128.546  6013911.480       107  62778.214  173801.613

TIDAL       2121772.462  5993470.060       201  98991.152  152599.533

SLOAT       2095678.561  5984226.406       202  72708.341  143900.815

ARMY        2100667.364  6012652.104       203  78288.679  172218.652

TRANSFORMATION SOLUTION RESIDUALS (Feet)

1999           2013      North     East    N.Azim & Dist

CANDLESTICK     107     -0.020    +0.016     140° 0.026

TIDAL           201     -0.032    -0.009     195° 0.033

SLOAT           202     +0.034    -0.012     340° 0.036

ARMY            203     +0.018    +0.004      13° 0.018

Root Mean Square of the North and East Residuals =    0.02

Scale Factor =  1.00007856      Standard Deviation =  0.00000079

Rotation     = -1° 11' 37.4"    Standard Deviation =  0° 00' 00.2"

TRANSFORMATION EQUATIONS   N2=A1*N1-A2*E1+A4   E2=A2*N1+A1*E1+A3

A1= 0.9998615081  A2= -0.0208347280  A3= -5795833.91914  A4= -2147359.74825

RECOMMENDATION - SUMMARY

As stated previously, CCSF sits between two major faults and it is expected that future re-surveys of the HPN will be conducted to determine secular and episodic movements within the area. If future surveys of the HPN follow the specifications and procedures adopted for this survey, the relative accuracy of measured shifts is expected to approach 5-6 millimeters at the 95% level of confidence. Statistically, this means the probability at the 95% level of confidence is that movement (signal) has occurred if the movement between two epochs is greater than the relative 95% error (noise). The differences in successive coordinates could be used to estimate movements but do not provide statistical information about the relative accuracies of movements. Measurements of temporal movements should be based on a rigorous simultaneous least squares adjustment of multiple redundant observations at two different epochs for each point to compute the movement and relative accuracy. The success of this process is predicated on the absolute recovery of the reference frame in this survey which is the IGS08(2005) 2013.54 Epoch based on the National Spatial Reference System (NSRS) CORS.

APPENDIX

GLOSSARY

CCSF		City and County of San Francisco

CCSF-CS	City and County of San Francisco Coordinate System

CGPS		Continuously GPS Stations in California

CORS		Continuously Operated Reference Stations in the National Spatial Reference System

FGCC		Federal Geodetic Control Committee

FGDC		Federal Geographic Data Committee

GIS		Geographic Information System

GLONASS	Globalnaya Navigatsionnaya Sputnikovaya Sistema (Russian GPS system)

GNSS		Global Navigation Satellite System (GPS and GLONASS used in this survey)

GPS		Global Positioning System

HPN		High Precision Network

IGS08 		International GNSS Service 2008 Reference Frame

K INDEX	Space weather index ranging from 0-9, 1 being calm and 5 indicating a geomagnetic storm

LDP		Low Distortion Projection

NAD83		North American Datum of 1983

NAVD88	North American Vertical Datum of 1988

NGS		National Geodetic Survey

RTN		Real-Time Network

SOPAC		Scripps Orbit and Permanent Array Center

GDOP		Geometric Dilution of Precision

L1		Carrier frequency @ 1575.42 MHz

L2		Carrier frequency @1227.60 MHz

NAVD29	North American Vertical Datum of 1929

GRS80		Geodetic Reference System of 1980

HTDP		Horizontal Time-Dependent Positioning Program

PLS		Professional Land Surveyor

NOAA		National Oceanic and Atmospheric Administration

“2013 NAD83(2011) 2010.00 Epoch” represents the 2011 Adjustment of the North American Datum at Epoch 2010.00 established in CCSF in 2013

NAD83 GEODETIC COORDINATE LIST

DATUM & REFERENCE FRAME: NAD83(2011) Epoch 2010.00

POINT    N. LATITUDE        W. LONGITUDE     ELLIPSOID HT   ELLIPSOID HT

‘  “              ‘  “            meters          feet

101     37-45-54.52969    122-26-21.55646     118.188        387.754

102     37-44-16.70006    122-26-35.16804     138.344        453.883

103     37-46-48.82346    122-30-42.69982      13.592         44.594

104     37-45-03.75809    122-30-29.78795     -25.278        -82.931

105     37-42-54.76774    122-30-04.94280      23.607         77.449

106     37-42-25.30199    122-27-09.62037      77.575        254.510

107     37-42-22.15117    122-23-36.90485     -28.944        -94.959

108     37-44-24.76543    122-22-36.10746     -28.109        -92.221

109     37-46-35.29523    122-23-14.65957     -29.098        -95.467

110     37-48-32.30625    122-24-49.85029     -29.261        -96.002

111     37-48-21.50380    122-28-02.95054     -28.605        -93.848

112     37-47-16.23698    122-28-31.85392      21.692         71.168

113     37-46-15.21109    122-28-05.24080      42.159        138.316

114     37-45-06.84452    122-29-07.34048      66.915        219.538

115     37-43-59.89229    122-28-41.12920      28.692         94.135

116     37-43-33.80444    122-27-19.05616      57.292        187.966

117     37-43-04.59553    122-24-45.07281      84.526        277.316

118     37-44-35.20539    122-24-35.75058      45.947        150.745

119     37-46-46.37693    122-25-01.43682     -13.634        -44.731

120     37-47-27.12564    122-26-15.67191      53.304        174.881

CCSF    37-44-55.63969    122-24-01.58310     -15.938        -52.290

MHDL    37-50-32.34805    122-29-39.54740      66.402        217.855

UCSF    37-45-46.67199    122-27-29.28890     155.128        508.949

IGS08 GEODETIC COORDINATE LIST

DATUM & REFERENCE FRAME: IGS08(2005) Epoch 2013.54

POINT   N. LATITUDE        W. LONGITUDE    ELLIPSOID HT

‘  “              ‘  “           meters

101     37-45-54.54414    122-26-21.61608     117.665

102     37-44-16.71449    122-26-35.22764     137.821

103     37-46-48.83787    122-30-42.75949      13.072

104     37-45-03.77249    122-30-29.84759     -25.798

105     37-42-54.78214    122-30-05.00241      23.085

106     37-42-25.31641    122-27-09.67995      77.053

107     37-42-22.16562    122-23-36.96440     -29.467

108     37-44-24.77990    122-22-36.16703     -28.633

109     37-46-35.30971    122-23-14.71918     -29.621

110     37-48-32.32072    122-24-49.90994     -29.783

111     37-48-21.51824    122-28-03.01021     -29.126

112     37-47-16.25141    122-28-31.91358      21.171

113     37-46-15.22552    122-28-05.30044      41.637

114     37-45-06.85894    122-29-07.40011      66.394

115     37-43-59.90670    122-28-41.18882      28.171

116     37-43-33.81887    122-27-19.11576      56.770

117     37-43-04.60998    122-24-45.13238      84.003

118     37-44-35.21984    122-24-35.81017      45.424

119     37-46-46.39139    122-25-01.49644     -14.157

120     37-47-27.14010    122-26-15.73155      52.782

CCSF    37-44-55.65415    122-24-01.64269     -16.461

EBMD    37-48-54.03039    122-17-01.70688     -15.922

MHDL    37-50-32.36248    122-29-39.60710      65.888

P176    37-28-18.38315    122-21-25.71047     433.810

P178    37-32-04.26983    122-19-56.51784     129.049

P224    37-51-50.02759    122-13-08.62234     407.349

TIBB    37-53-27.15354    122-26-51.37682     -21.084

UCSF    37-45-46.68643    122-27-29.34853     154.606

WINT    37-39-09.51974    122-08-26.05320     -28.790

PLANE COORDINATE LIST

California State Plane Coordinates Zone 3: NAD83(2011) 2010.00 Epoch and NAVD88 Heights per CCSF 2013 2nd Order Leveling Survey

POINT#    STATE PLANE COORD.     NAVD88       STATE PLANE COORD.       NAVD88    Convergence         ----- Factors -----

NORTH(m)    EAST(m)    HEIGHT(m)    NORTH(ft)    EAST(ft)    HEIGHT(ft)    Angle       Scale   x Ellipsoid = Combined

101    642177.520  1829147.188   150.799    2106877.414  6001127.064   494.746   -1-11-14.3   0.99992921  0.99998145  0.99991066

102    639169.103  1828751.516   170.991    2097007.299  5999828.933   560.993   -1-11-22.7   0.99992920  0.99997829  0.99990750

103    643985.863  1822793.428    46.352    2112810.286  5980281.439   152.073   -1-13-54.2   0.99992931  0.99999787  0.99992718

104    640740.769  1823039.791     7.550    2102163.674  5981089.714    24.770   -1-13-46.3   0.99992918  1.00000397  0.99993315

105    636752.037  1823562.786    56.489    2089077.308  5982805.572   185.331   -1-13-31.1   0.99992937  0.99999630  0.99992567

106    635753.146  1827836.561   110.302    2085800.113  5996827.116   361.882   -1-11-43.8   0.99992947  0.99998783  0.99991730

107    635548.974  1833043.520     3.698    2085130.260  6013910.280    12.133   -1-09-33.5   0.99992948  1.00000454  0.99993403

108    639298.261  1834608.138     4.484    2097431.045  6019043.533    14.711   -1-08-56.3   0.99992920  1.00000441  0.99993361

109    643340.513  1833745.648     3.461    2110693.000  6016213.847    11.355   -1-09-19.9   0.99992928  1.00000457  0.99993384

110    646994.405  1831490.574     3.279    2122680.811  6008815.324    10.758   -1-10-18.2   0.99992969  1.00000459  0.99993428

111    646759.384  1826761.484     4.000    2121909.745  5993299.969    13.123   -1-12-16.4   0.99992964  1.00000449  0.99993412

112    644762.606  1826012.184    54.344    2115358.651  5990841.639   178.294   -1-12-34.1   0.99992938  0.99999660  0.99992598

113    642867.936  1826623.601    74.816    2109142.554  5992847.597   245.459   -1-12-17.8   0.99992924  0.99999338  0.99992262

114    640792.840  1825059.534    99.656    2102334.511  5987716.154   326.955   -1-12-55.8   0.99992918  0.99998950  0.99991868

115    638715.658  1825657.369    61.448    2095519.621  5989677.550   201.601   -1-12-39.8   0.99992923  0.99999550  0.99992472

116    637869.351  1827649.621    89.985    2092743.029  5996213.798   295.226   -1-11-49.5   0.99992927  0.99999101  0.99992028

117    636891.154  1831400.952   117.172    2089533.729  6008521.289   384.422   -1-10-15.3   0.99992935  0.99998674  0.99991608

118    639679.300  1831686.204    78.553    2098681.170  6009457.154   257.719   -1-10-09.5   0.99992919  0.99999279  0.99992198

119    643735.185  1831140.342    18.941    2111987.853  6007666.271    62.142   -1-10-25.3   0.99992930  1.00000214  0.99993144

120    645028.567  1829350.275    85.887    2116231.223  6001793.361   281.781   -1-11-10.7   0.99992942  0.99999164  0.99992106

201    646718.080  1826812.971     3.659    2121774.233  5993468.889    12.005   -1-12-15.1   0.99992963  1.00000454  0.99993417

202    638764.662  1823995.481    23.761    2095680.395  5984225.175    77.956   -1-13-21.4   0.99992923  1.00000142  0.99993065

203    640285.230  1832659.665              2100669.127  6012650.919             -1-09-45.5   0.99992918  1.00000434  0.99993352

CCSF   640292.109  1832535.278              2100691.693  6012242.824             -1-09-48.6   0.99992918  1.00000250  0.99993168

MHDL   650842.310  1824485.178              2135305.146  5985831.789             -1-13-15.5   0.99993044  0.99998958  0.99992002

UCSF   641969.844  1827484.810    187.77    2106196.063  5995673.081   616.04    -1-11-55.8   0.99992920  0.99997566  0.99990486

Average -1-11-33.7   0.99992936  0.99999560  0.99992496

CCSF Plane Coordinates: NAD83(2011) 2010.00 Epoch and NAVD88 Heights per CCSF 2013 2nd Order Leveling Survey

POINT#    CCSF PLANE COORD.     NAVD88       CCSF PLANE COORD.     NAVD88    Convergence         ----- Factors -----

NORTH(m)   EAST(m)    HEIGHT(m)    NORTH(ft)  EAST(ft)   HEIGHT(ft)    Angle       Scale   x Ellipsoid = Combined

101     25681.275  48940.932   150.799    84255.983   160567.041   494.746    0-00-23.5   1.00000701  0.99998145  0.99998847

102     22665.032  48608.002   170.991    74360.193   159474.752   560.993    0-00-15.2   1.00000700  0.99997829  0.99998530

103     27356.975  42550.378    46.352    89753.676   139600.697   152.073   -0-02-16.4   1.00000737  0.99999787  1.00000523

104     24117.466  42864.322     7.550    79125.385   140630.698    24.770   -0-02-08.4   1.00000732  1.00000397  1.00001129

105     20140.186  43470.358    56.489    66076.592   142618.999   185.331   -0-01-53.1   1.00000725  0.99999630  1.00000355

106     19230.486  47764.351   110.302    63092.021   156706.874   361.882   -0-00-05.9   1.00000700  0.99998783  0.99999483

107     19134.838  52974.838     3.698    62778.214   173801.613    12.133    0-02-04.2   1.00000730  1.00000454  1.00001185

108     22916.205  54461.117     4.484    75184.248   178677.849    14.711    0-02-41.5   1.00000751  1.00000441  1.00001192

109     26939.924  53514.519     3.461    88385.400   175572.218    11.355    0-02-18.0   1.00000737  1.00000457  1.00001194

110     30546.317  51183.622     3.279   100217.376   167924.933    10.758    0-01-19.8   1.00000712  1.00000459  1.00001172

111     30212.789  46460.090     4.000    99123.126   152427.813    13.123   -0-00-38.6   1.00000703  1.00000449  1.00001152

112     28200.678  45752.500    54.344    92521.723   150106.327   178.294   -0-00-56.3   1.00000706  0.99999660  1.00000366

113     26319.011  46403.311    74.816    86348.290   152241.529   245.459   -0-00-40.0   1.00000703  0.99999338  1.00000042

114     24211.614  44882.700    99.656    79434.271   147252.657   326.955   -0-01-18.0   1.00000712  0.99998950  0.99999662

115     22147.178  45523.733    61.448    72661.199   149355.781   201.601   -0-01-01.9   1.00000708  0.99999550  1.00000257

116     21342.501  47533.342    89.985    70021.189   155948.974   295.226   -0-00-11.7   1.00000700  0.99999101  0.99999801

117     20442.606  51304.532   117.172    67068.783   168321.620   384.422    0-01-22.5   1.00000713  0.99998674  0.99999387

118     23236.307  51531.650    78.553    76234.451   169066.754   257.719    0-01-28.3   1.00000715  0.99999279  0.99999994

119     27280.254  50901.352    18.941    89501.966   166998.851    62.142    0-01-12.6   1.00000710  1.00000214  1.00000924

120     28536.156  49084.584    85.887    93622.371   161038.340   281.781    0-00-27.1   1.00000701  0.99999164  0.99999865

201     30172.563  46512.431     3.659    98991.152   152599.533    12.005   -0-00-37.3   1.00000703  1.00000454  1.00001157

202     22161.547  43861.056    23.761    72708.341   143900.815    77.956   -0-01-43.4   1.00000721  1.00000142  1.00000863

203     23862.437  52492.350              78288.679   172218.652              0-01-52.3   1.00000725  1.00000434  1.00001159

CCSF    23866.723  52367.837              78302.739   171810.144              0-01-49.2   1.00000723  1.00000250  1.00000974

MHDL    34247.715  44099.030             112361.043   144681.568             -0-01-37.9   1.00000719  0.99998958  0.99999677

UCSF    25438.989  47283.113   187.77     83461.083   155128.014   616.04    -0-00-17.9   1.00000701  0.99997566  0.99998266

Average 0-00-04.1   1.00000715  0.99999560  1.00000275

Projection Specifications:

Projection: Transverse Mercator, Ellipsoid: GRS-80, Scale: 1.000007, Latitude of Origin: 37º45’00”,

Central Meridian: -122º27’00”, False Northing: 24000.0 meters, False Easting: 48000.0 meters.

City & County of San Francisco Regional Network & High Precision Network (north up)

City & County of San Francisco High Precision Network

(north up)

NGS STATION DESCRIPTIONS  - CORS NGS Reference Data for P176, P224, TIBB & WINT

CORS Coordinates, HTDP Solutions for NAD83(2011) 2010.00 to 2013.54, and NGS Data Sheets (retrieved at the time of this survey)and short-term time series graphs

P176

______________________________________________________________________________

MILLSCREEKCN2007 (P176),  CALIFORNIA

|                             PID = DN7542                                    |

| IGS08 VELOCITY                                                              |

| Predicted with HTDP_3.1.2 Apr 2012.                                         |

|     VX =  -0.0237 m/yr      northward =   0.0213 m/yr                       |

|     VY =   0.0303 m/yr      eastward  =  -0.0362 m/yr                       |

|     VZ =   0.0169 m/yr      upward    =   0.0000 m/yr                       |

|                                                                             |

| NAD_83 (2011) VELOCITY                                                      |

| Transformed from IGS08 velocity in Apr 2012.                                |

|     VX =  -0.0074 m/yr      northward =   0.0344 m/yr                       |

|     VY =   0.0307 m/yr      eastward  =  -0.0227 m/yr                       |

|     VZ =   0.0265 m/yr      upward    =  -0.0013 m/yr                       |

|_____________________________________________________________________________|

|                                                                             |

|         Monument: MILLSCREEKCN2007 GRP                                      |

|         ------------------------------                                      |

|                             PID = DN7544                                    |

| IGS08 POSITION (EPOCH 2005.0)                                               |

| Computed in Apr 2012 using 29 days of data.                                 |

|     X =  -2712719.288 m     latitude    =  37 28 18.37726 N                 |

|     Y =  -4281638.058 m     longitude   = 122 21 25.69787 W                 |

|     Z =   3859341.975 m     ellipsoid height =  433.810   m                 |

|                                                                             |

| NAD_83 (2011) POSITION (EPOCH 2010.0)                                       |

| Transformed from IGS08 (epoch 2005.0) position in Apr 2012.                 |

|     X =  -2712718.623 m     latitude    =  37 28 18.36834 N                 |

|     Y =  -4281639.174 m     longitude   = 122 21 25.65069 W                 |

|     Z =   3859342.078 m     ellipsoid height =  434.339   m                 |

|                                                                             |

* The position & velocity were revised in Apr 2012.

______________________________________________________________________________

HTDP (version 3.2.3) OUTPUT

HTDP UPDATED POSITIONS IN NAD_83(2011/CORS96/2007)

FROM  1-01-2010 TO  7-17-2013 (month-day-year)

FROM 2010.000 TO 2013.541 (decimal years)

OLD COORDINATE    NEW COORDINATE    VELOCITY      DISPLACEMENT

P176

LATITUDE     37 28 18.36834 N   37 28 18.37230 N   34.53 mm/yr   0.122 m north

LONGITUDE   122 21 25.65069 W  122 21 25.65398 W  -22.87 mm/yr  -0.081 m east

ELLIP. HT.            434.339            434.334   -1.35 mm/yr  -0.005 m up

X                -2712718.623       -2712718.650   -7.50 mm/yr  -0.027 m

Y                -4281639.174       -4281639.065   30.89 mm/yr   0.109 m

Z                 3859342.078        3859342.172   26.58 mm/yr   0.094 m

______________________________________________________________________________

DN7544 NGS Data Sheet *******************************************************************

DN7544  CORS        -  This is a GPS Continuously Operating Reference Station.

DN7544  DESIGNATION -  MILLSCREEKCN2007 GRP

DN7544  CORS_ID     -  P176

DN7544  PID         -  DN7544

DN7544  STATE/COUNTY-  CA/SAN MATEO

DN7544  COUNTRY     -  US

DN7544  USGS QUAD   -  WOODSIDE (1997)

DN7544                         *CURRENT SURVEY CONTROL

DN7544  ______________________________________________________________________

DN7544* NAD 83(2011) POSITION- 37 28 18.36834(N) 122 21 25.65069(W)   ADJUSTED

DN7544* NAD 83(2011) ELLIP HT-   434.339 (meters)        (04/??/12)   ADJUSTED

DN7544* NAD 83(2011) EPOCH   -  2010.00

DN7544* NAVD 88 ORTHO HEIGHT -          **(meters)           **(feet)

DN7544  ______________________________________________________________________

DN7544  NAD 83(2011) X  - -2,712,718.623 (meters)                     COMP

DN7544  NAD 83(2011) Y  - -4,281,639.174 (meters)                     COMP

DN7544  NAD 83(2011) Z  -  3,859,342.078 (meters)                     COMP

DN7544  LAPLACE CORR    -          5.29  (seconds)                    DEFLEC12A

DN7544  GEOID HEIGHT    -        -32.59  (meters)                     GEOID12A

DN7544  HORZ ORDER      -  SPECIAL (CORS)

DN7544  ELLP ORDER      -  SPECIAL (CORS)

DN7544

DN7544.Formal positional accuracy estimates are not available for this CORS

DN7544.because its coordinates were determined in part using modeled

DN7544.velocities. Approximate one-sigma accuracies for latitude, longitude,

DN7544.and ellipsoid height can be obtained from the short-term time series.

DN7544.Additional information regarding modeled velocities is available on

DN7544.the CORS Coordinates and Multi-Year CORS Solution FAQ web pages.

DN7544

DN7544.The horizontal coordinates were established by GPS observations

DN7544.and adjusted by the National Geodetic Survey in April 2012.

DN7544

DN7544.NAD 83(2011) refers to NAD 83 coordinates where the reference

DN7544.frame has been affixed to the stable North American Tectonic Plate.

DN7544

DN7544.The horizontal coordinates are valid at the epoch date displayed above

DN7544.which is a decimal equivalence of Year/Month/Day.

DN7544

DN7544.The XYZ, and position/ellipsoidal ht. are equivalent.

DN7544

DN7544.The Laplace correction was computed from DEFLEC12A derived deflections.

DN7544

DN7544.The ellipsoidal height was determined by GPS observations

DN7544.and is referenced to NAD 83.

DN7544

DN7544. The following values were computed from the NAD 83(2011) position.

DN7544

DN7544;                    North         East     Units Scale Factor Converg.

DN7544;SPC CA 3     -   609,477.407 1,835,741.550   MT  0.99994099   -1 08 13.2

DN7544;SPC CA 3     - 1,999,593.79  6,022,762.07   sFT  0.99994099   -1 08 13.2

DN7544;UTM  10      - 4,147,403.346   556,845.451   MT  0.99963980   +0 23 28.0

DN7544

DN7544!             -  Elev Factor  x  Scale Factor =   Combined Factor

DN7544!SPC CA 3     -   0.99993185  x   0.99994099  =   0.99987284

DN7544!UTM  10      -   0.99993185  x   0.99963980  =   0.99957167

DN7544

DN7544                          SUPERSEDED SURVEY CONTROL

DN7544

DN7544  NAD 83(CORS)-  37 28 18.35942(N)    122 21 25.64286(W) AD(2002.00) A

DN7544  ELLIP H (04/??/12)  434.346  (m)                       GP(2002.00) 4 1

DN7544

DN7544.Superseded values are not recommended for survey control.

DN7544

DN7544.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

DN7544.See file dsdata.txt to determine how the superseded data were derived.

DN7544

DN7544_U.S. NATIONAL GRID SPATIAL ADDRESS: 10SEG5684547403(NAD 83)

DN7544

DN7544_STAMPING: UNKNOWN

DN7544                          STATION DESCRIPTION

DN7544'THIS MONUMENT IS ASSOCIATED WITH CORS SITE 'P176'

DN7544'LATEST INFORMATION INCLUDING POSITIONS AND VELOCITIES

DN7544'ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

DN7544'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

DN7544'   ftp://cors.ngs.noaa.gov/cors/README.txt

DN7544'   ftp://cors.ngs.noaa.gov/cors/coord/coord_08

DN7544'   ftp://cors.ngs.noaa.gov/cors/station_log

DN7544'   http://geodesy.noaa.gov/CORS

P224

______________________________________________________________________________

SIBLEYVOLCCN2005 (P224),  CALIFORNIA

| IGS08 VELOCITY                                                              |

| Computed in Aug 2011 using data through gpswk 1631.                         |

|     VX =  -0.0211 m/yr      northward =   0.0068 m/yr                       |

|     VY =   0.0181 m/yr      eastward  =  -0.0275 m/yr                       |

|     VZ =   0.0054 m/yr      upward    =   0.0001 m/yr                       |

| NAD_83 (2011) VELOCITY                                                      |

| Transformed from IGS08 velocity in Aug 2011.                                |

|     VX =  -0.0047 m/yr      northward =   0.0198 m/yr                       |

|     VY =   0.0185 m/yr      eastward  =  -0.0138 m/yr                       |

|     VZ =   0.0149 m/yr      upward    =  -0.0012 m/yr                       |

|_____________________________________________________________________________|

|         Monument: SIBLEYVOLCCN2005 GRP                                      |

|         ------------------------------                                      |

| IGS08 POSITION (EPOCH 2005.0)                                               |

| Computed in Aug 2011 using data through gpswk 1631.                         |

|     X =  -2688201.251 m     latitude    =  37 51 50.02572 N                 |

|     Y =  -4265643.690 m     longitude   = 122 13 08.61274 W                 |

|     Z =   3893778.570 m     ellipsoid height =  407.348   m                 |

|                                                                             |

| NAD_83 (2011) POSITION (EPOCH 2010.0)                                       |

| Transformed from IGS08 (epoch 2005.0) position in Aug 2011.                 |

|     X =  -2688200.572 m     latitude    =  37 51 50.01427 N                 |

|     Y =  -4265644.864 m     longitude   = 122 13 08.56363 W                 |

|     Z =   3893778.614 m     ellipsoid height =  407.873   m                 |

* The position & velocity were revised in Aug 2011.

______________________________________________________________________________

HTDP (version 3.2.3) OUTPUT

UPDATED POSITIONS IN NAD_83(2011/CORS96/2007)

FROM  1-01-2010 TO  7-17-2013 (month-day-year)

FROM 2010.000 TO 2013.541 (decimal years)

OLD COORDINATE    NEW COORDINATE    VELOCITY      DISPLACEMENT

P224

LATITUDE     37 51 50.01427 N   37 51 50.01666 N   20.82 mm/yr   0.074 m north

LONGITUDE   122 13  8.56363 W  122 13  8.56557 W  -13.38 mm/yr  -0.047 m east

ELLIP. HT.            407.873            407.868   -1.36 mm/yr  -0.005 m up

X                -2688200.572       -2688200.586   -3.94 mm/yr  -0.014 m

Y                -4265644.864       -4265644.797   18.85 mm/yr   0.067 m

Z                 3893778.613        3893778.669   15.60 mm/yr   0.055 m

Used HTDP and the CORS velocities to compute the 2013.54 Epoch below

OLD COORDINATE    NEW COORDINATE    VELOCITY      DISPLACEMENT

P224

LATITUDE     37 51 50.01427 N   37 51 50.01654 N   19.80 mm/yr   0.070 m north

LONGITUDE   122 13  8.56363 W  122 13  8.56563 W  -13.80 mm/yr  -0.049 m east

ELLIP. HT.            407.873            407.869   -1.20 mm/yr  -0.004 m up

X                -2688200.572       -2688200.589   -4.69 mm/yr  -0.017 m

Y                -4265644.864       -4265644.799   18.44 mm/yr   0.065 m

Z                 3893778.613        3893778.666   14.89 mm/yr   0.053 m

______________________________________________________________________________

DH3881 NGS Data Sheet **********************************************************************

DH3881  CORS        -  This is a GPS Continuously Operating Reference Station.

DH3881  DESIGNATION -  SIBLEYVOLCCN2005 GRP

DH3881  CORS_ID     -  P224

DH3881  PID         -  DH3881

DH3881  STATE/COUNTY-  CA/CONTRA COSTA

DH3881  COUNTRY     -  US

DH3881  USGS QUAD   -  OAKLAND EAST (1997)

DH3881                         *CURRENT SURVEY CONTROL

DH3881  ______________________________________________________________________

DH3881* NAD 83(2011) POSITION- 37 51 50.01427(N) 122 13 08.56363(W)   ADJUSTED

DH3881* NAD 83(2011) ELLIP HT-   407.873 (meters)        (08/??/11)   ADJUSTED

DH3881* NAD 83(2011) EPOCH   -  2010.00

DH3881* NAVD 88 ORTHO HEIGHT -          **(meters)           **(feet)

DH3881  ______________________________________________________________________

DH3881  NAD 83(2011) X  - -2,688,200.572 (meters)                     COMP

DH3881  NAD 83(2011) Y  - -4,265,644.864 (meters)                     COMP

DH3881  NAD 83(2011) Z  -  3,893,778.614 (meters)                     COMP

Laplace CORR    -          2.51  (seconds)                    DEFLEC12A

DH3881  GEOID HEIGHT    -        -32.17  (meters)                     GEOID12A

DH3881  HORZ ORDER      -  SPECIAL (CORS)

DH3881  ELLP ORDER      -  SPECIAL (CORS)

DH3881

DH3881  FGDC Geospatial Positioning Accuracy Standards (95% confidence, cm)

DH3881  Type                                         Horiz  Ellip  Dist(km)

DH3881  -------------------------------------------------------------------

DH3881  NETWORK                                       2.02   6.62

DH3881  -------------------------------------------------------------------

DH3881  NOTE: Click here for information on individual local accuracy

DH3881  values and other accuracy information.

DH3881 ********************************************************************

DH3881  ACCURACIES  -  Complete network and local accuracy information.

DH3881  CORS        -  This is a GPS Continuously Operating Reference Station.

DH3881  DESIGNATION -  SIBLEYVOLCCN2005 GRP

DH3881  PID         -  DH3881

DH3881

DH3881  Statistical Information, in cm, for point DH3881 follows.

DH3881

DH3881  Note that Horz and Ellip values are the official 95%

DH3881  FGDC accuracy standards.  The values of StdN, StdE and Stdh are the

DH3881  standard deviations (one sigma) of the coordinates (NETWORK) or

DH3881  of the difference in the coordinates (LOCAL) in Latitude, Longitude

DH3881  and Ellipsoid Height.  The value CorrNE is the correlation

DH3881  coefficient between the latitude and longitude components of either

DH3881  the coordinate (NETWORK) or coordinate difference (LOCAL).

DH3881

DH3881  Type/PID  Horz  Ellip  Dist(km)   StdN   StdE   Stdh         CorrNE

DH3881  -------------------------------------------------------------------

DH3881  NETWORK   2.02   6.62     0.00    0.91   0.71   3.38    +0.04283161

DH3881  -------------------------------------------------------------------

DH3881

DH3881.The horizontal coordinates were established by GPS observations

DH3881.and adjusted by the National Geodetic Survey in August 2011.

DH3881

DH3881.NAD 83(2011) refers to NAD 83 coordinates where the reference

DH3881.frame has been affixed to the stable North American Tectonic Plate.

DH3881

DH3881.The horizontal coordinates are valid at the epoch date displayed above

DH3881.which is a decimal equivalence of Year/Month/Day.

DH3881

DH3881.The XYZ, and position/ellipsoidal ht. are equivalent.

DH3881

DH3881.The Laplace correction was computed from DEFLEC12A derived deflections.

DH3881

DH3881.The ellipsoidal height was determined by GPS observations

DH3881.and is referenced to NAD 83.

DH3881

DH3881. The following values were computed from the NAD 83(2011) position.

DH3881

DH3881;                    North         East     Units Scale Factor Converg.

DH3881;SPC CA 3     -   652,755.675 1,848,752.302   MT  0.99993111   -1 03 08.8

DH3881;SPC CA 3     - 2,141,582.58  6,065,448.18   sFT  0.99993111   -1 03 08.8

DH3881;UTM  10      - 4,191,001.178   568,692.783   MT  0.99965812   +0 28 45.7

DH3881

DH3881!             -  Elev Factor  x  Scale Factor =   Combined Factor

DH3881!SPC CA 3     -   0.99993600  x   0.99993111  =   0.99986712

DH3881!UTM  10      -   0.99993600  x   0.99965812  =   0.99959414

DH3881

DH3881                          SUPERSEDED SURVEY CONTROL

DH3881

DH3881  NAD 83(CORS)-  37 51 50.00906(N)    122 13 08.55850(W) AD(2002.00) A

DH3881  ELLIP H (06/??/05)  407.864  (m)                       GP(2002.00) 4 1

DH3881

DH3881.Superseded values are not recommended for survey control.

DH3881

DH3881.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

DH3881.See file dsdata.txt to determine how the superseded data were derived.

DH3881

DH3881_U.S. NATIONAL GRID SPATIAL ADDRESS: 10SEG6869291001(NAD 83)

DH3881

DH3881_STAMPING: UNKNOWN

DH3881

DH3881                          STATION DESCRIPTION

DH3881

DH3881'THIS MONUMENT IS ASSOCIATED WITH CORS SITE 'P224'

DH3881'LATEST INFORMATION INCLUDING POSITIONS AND VELOCITIES

DH3881'ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

DH3881'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

DH3881'   ftp://cors.ngs.noaa.gov/cors/README.txt

DH3881'   ftp://cors.ngs.noaa.gov/cors/coord/coord_08

DH3881'   ftp://cors.ngs.noaa.gov/cors/station_log

DH3881'   http://geodesy.noaa.gov/CORS

TIBB

______________________________________________________________________________

TIBURON PENINSUL (TIBB),  CALIFORNIA

| IGS08 VELOCITY                                                              |

| Predicted with HTDP_3.2.3 Dec 2012.                                         |

|     VX =  -0.0216 m/yr      northward =   0.0148 m/yr                       |

|     VY =   0.0245 m/yr      eastward  =  -0.0314 m/yr                       |

|     VZ =   0.0117 m/yr      upward    =   0.0000 m/yr                       |

|                                                                             |

| NAD_83 (2011) VELOCITY                                                      |

| Transformed from IGS08 velocity in Dec 2012.                                |

|     VX =  -0.0052 m/yr      northward =   0.0280 m/yr                       |

|     VY =   0.0249 m/yr      eastward  =  -0.0177 m/yr                       |

|     VZ =   0.0213 m/yr      upward    =  -0.0013 m/yr                       |

|_____________________________________________________________________________|

|         Monument: TIBURON PENINSUL GRP                                      |

|         ------------------------------                                      |

|                             PID = AI4507                                    |

| IGS08 POSITION (EPOCH 2005.0)                                               |

| Computed in Dec 2012 using 13 days of data.                                 |

|     X =  -2704026.807 m     latitude    =  37 53 27.14944 N                 |

|     Y =  -4253050.099 m     longitude   = 122 26 51.36586 W                 |

|     Z =   3895879.226 m     ellipsoid height =  -21.084   m                 |

|                                                                             |

| NAD_83 (2011) POSITION (EPOCH 2010.0)                                       |

| Transformed from IGS08 (epoch 2005.0) position in Dec 2012.                 |

|     X =  -2704026.130 m     latitude    =  37 53 27.13938 N                 |

|     Y =  -4253051.240 m     longitude   = 122 26 51.31741 W                 |

|     Z =   3895879.300 m     ellipsoid height =  -20.565   m                 |

|                                                                             |

______________________________________________________________________________

HTDP (version 3.2.3) OUTPUT

UPDATED POSITIONS IN NAD_83(2011/CORS96/2007)

FROM  1-01-2010 TO  7-17-2013 (month-day-year)

FROM 2010.000 TO 2013.541 (decimal years)

OLD COORDINATE    NEW COORDINATE    VELOCITY      DISPLACEMENT

TIBB

LATITUDE     37 53 27.13938 N   37 53 27.14260 N   28.06 mm/yr   0.099 m north

LONGITUDE   122 26 51.31741 W  122 26 51.31998 W  -17.72 mm/yr  -0.063 m east

ELLIP. HT.            -20.565            -20.570   -1.36 mm/yr  -0.005 m up

X                -2704026.130       -2704026.148   -5.13 mm/yr  -0.018 m

Y                -4253051.240       -4253051.152   24.96 mm/yr   0.088 m

Z                 3895879.300        3895879.376   21.31 mm/yr   0.075 m

______________________________________________________________________________

AI4507 NGS Data Sheet *******************************************************************

AI4507  CORS        -  This is a GPS Continuously Operating Reference Station.

AI4507  DESIGNATION -  TIBURON PENINSUL GRP

AI4507  CORS_ID     -  TIBB

AI4507  PID         -  AI4507

AI4507  STATE/COUNTY-  CA/MARIN

AI4507  COUNTRY     -  US

AI4507  USGS QUAD   -  SAN QUENTIN (1995)

AI4507                         *CURRENT SURVEY CONTROL

AI4507  ______________________________________________________________________

AI4507* NAD 83(2011) POSITION- 37 53 27.13938(N) 122 26 51.31741(W)   ADJUSTED

AI4507* NAD 83(2011) ELLIP HT-   -20.565 (meters)        (12/??/12)   ADJUSTED

AI4507* NAD 83(2011) EPOCH   -  2010.00

AI4507* NAVD 88 ORTHO HEIGHT -    11.8    (meters)      39.    (feet) GPS OBS

AI4507  ______________________________________________________________________

AI4507  NAD 83(2011) X  - -2,704,026.130 (meters)                     COMP

AI4507  NAD 83(2011) Y  - -4,253,051.240 (meters)                     COMP

AI4507  NAD 83(2011) Z  -  3,895,879.300 (meters)                     COMP

AI4507  LAPLACE CORR    -          0.01  (seconds)                    DEFLEC12A

AI4507  GEOID HEIGHT    -        -32.38  (meters)                     GEOID12A

AI4507  HORZ ORDER      -  SPECIAL (CORS)

AI4507  ELLP ORDER      -  SPECIAL (CORS)

AI4507

AI4507.Formal positional accuracy estimates are not available for this CORS

AI4507.because its coordinates were determined in part using modeled

AI4507.velocities. Approximate one-sigma accuracies for latitude, longitude,

AI4507.and ellipsoid height can be obtained from the short-term time series.

AI4507.Additional information regarding modeled velocities is available on

AI4507.the CORS Coordinates and Multi-Year CORS Solution FAQ web pages.

AI4507

AI4507.The horizontal coordinates were established by GPS observations

AI4507.and adjusted by the National Geodetic Survey in December 2012.

AI4507

AI4507.NAD 83(2011) refers to NAD 83 coordinates where the reference

AI4507.frame has been affixed to the stable North American Tectonic Plate.

AI4507

AI4507.The horizontal coordinates are valid at the epoch date displayed above

AI4507.which is a decimal equivalence of Year/Month/Day.

AI4507

AI4507.The orthometric height was determined by GPS observations and a

AI4507.high-resolution geoid model.

AI4507

AI4507.The XYZ, and position/ellipsoidal ht. are equivalent.

AI4507

AI4507.The Laplace correction was computed from DEFLEC12A derived deflections.

AI4507

AI4507.The ellipsoidal height was determined by GPS observations

AI4507.and is referenced to NAD 83.

AI4507

AI4507. The following values were computed from the NAD 83(2011) position.

AI4507

AI4507;                    North         East     Units Scale Factor Converg.

AI4507;SPC CA 6     - 1,151,818.729 1,453,439.357   MT  1.00316890   -3 24 20.5

AI4507;SPC CA 6     - 3,778,925.28  4,768,492.29   sFT  1.00316890   -3 24 20.5

AI4507;UTM  10      - 4,193,850.969   548,572.252   MT  0.99962906   +0 20 21.4

AI4507

AI4507!             -  Elev Factor  x  Scale Factor =   Combined Factor

AI4507!SPC CA 6     -   1.00000323  x   1.00316890  =   1.00317214

AI4507!UTM  10      -   1.00000323  x   0.99962906  =   0.99963229

AI4507

AI4507                          SUPERSEDED SURVEY CONTROL

AI4507

AI4507  NAVD 88 (04/06/00)   11.7    (m)  GEOID99 model used   GPS OBS

AI4507

AI4507.Superseded values are not recommended for survey control.

AI4507

AI4507.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

AI4507.See file dsdata.txt to determine how the superseded data were derived.

AI4507

AI4507_U.S. NATIONAL GRID SPATIAL ADDRESS: 10SEG4857293850(NAD 83)

AI4507

AI4507_MARKER: Z = SEE DESCRIPTION

AI4507_SETTING: 0 = UNSPECIFIED SETTING

AI4507_STAMPING: UNKNOWN

AI4507_MARK LOGO: NONE

AI4507_MAGNETIC: N = NO MAGNETIC MATERIAL

AI4507_STABILITY: A = MOST RELIABLE AND EXPECTED TO HOLD

AI4507+STABILITY: POSITION/ELEVATION WELL

AI4507_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

AI4507+SATELLITE: SATELLITE OBSERVATIONS - 1998

AI4507

AI4507  HISTORY     - Date     Condition        Report By

AI4507  HISTORY     - 1998     MONUMENTED       NGS

AI4507

AI4507                          STATION DESCRIPTION

AI4507'DESCRIBED BY NATIONAL GEODETIC SURVEY 1998

AI4507'THIS MONUMENT IS ASSOCIATED WITH CORS SITE 'TIBB'

AI4507'LATEST INFORMATION INCLUDING POSITIONS AND VELOCITIES

AI4507'ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

AI4507'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

AI4507'   ftp://cors.ngs.noaa.gov/cors/README.txt

AI4507'   ftp://cors.ngs.noaa.gov/cors/coord/coord_08

AI4507'   ftp://cors.ngs.noaa.gov/cors/station_log

AI4507'   http://geodesy.noaa.gov/CORS

WINT

______________________________________________________________________________

WINT_BARD_CN1991 (WINT),  CALIFORNIA

| IGS08 VELOCITY                                                              |

| Predicted with HTDP_3.2.3 Dec 2012.                                         |

|     VX =  -0.0222 m/yr      northward =   0.0125 m/yr                       |

|     VY =   0.0229 m/yr      eastward  =  -0.0310 m/yr                       |

|     VZ =   0.0099 m/yr      upward    =   0.0000 m/yr                       |

| NAD_83 (2011) VELOCITY                                                      |

| Transformed from IGS08 velocity in Dec 2012.                                |

|     VX =  -0.0058 m/yr      northward =   0.0255 m/yr                       |

|     VY =   0.0233 m/yr      eastward  =  -0.0173 m/yr                       |

|     VZ =   0.0194 m/yr      upward    =  -0.0013 m/yr                       |

|                                                                             |

|         Monument: WINT_BARD_CN1991 GRP                                      |

|         ------------------------------                                      |

|                             PID = AI4510                                    |

| IGS08 POSITION (EPOCH 2005.0)                                               |

| Computed in Dec 2012 using 16 days of data.                                 |

|     X =  -2689810.025 m     latitude    =  37 39 09.51629 N                 |

|     Y =  -4281187.927 m     longitude   = 122 08 26.04241 W                 |

|     Z =   3874973.479 m     ellipsoid height =  -28.791   m                 |

|                                                                             |

| NAD_83 (2011) POSITION (EPOCH 2010.0)                                       |

| Transformed from IGS08 (epoch 2005.0) position in Dec 2012.                 |

|     X =  -2689809.352 m     latitude    =  37 39 09.50579 N                 |

|     Y =  -4281189.079 m     longitude   = 122 08 25.99416 W                 |

|     Z =   3874973.548 m     ellipsoid height =  -28.259   m                 |

______________________________________________________________________________

HTDP (version 3.2.3) OUTPUT

UPDATED POSITIONS IN NAD_83(2011/CORS96/2007)

FROM  1-01-2010 TO  7-17-2013 (month-day-year)

FROM 2010.000 TO 2013.541 (decimal years)

OLD COORDINATE    NEW COORDINATE    VELOCITY      DISPLACEMENT

WINT

LATITUDE     37 39  9.50579 N   37 39  9.50872 N   25.54 mm/yr   0.090 m north

LONGITUDE   122 08 25.99416 W  122 08 25.99666 W  -17.33 mm/yr  -0.061 m east

ELLIP. HT.            -28.259            -28.264   -1.35 mm/yr  -0.005 m up

X                -2689809.352       -2689809.373   -5.80 mm/yr  -0.021 m

Y                -4281189.080       -4281188.997   23.34 mm/yr   0.083 m

Z                 3874973.548        3874973.617   19.40 mm/yr   0.069 m

______________________________________________________________________________

AI4510 NGS Data Sheet *******************************************************************

AI4510  HT_MOD      -  This is a Height Modernization Survey Station.

AI4510  CORS        -  This is a GPS Continuously Operating Reference Station.

AI4510  DESIGNATION -  WINT_BARD_CN1991 GRP

AI4510  CORS_ID     -  WINT

AI4510  PID         -  AI4510

AI4510  STATE/COUNTY-  CA/ALAMEDA

AI4510  COUNTRY     -  US

AI4510  USGS QUAD   -  SAN LEANDRO (1993)

AI4510                         *CURRENT SURVEY CONTROL

AI4510  ______________________________________________________________________

AI4510* NAD 83(2011) POSITION- 37 39 09.50579(N) 122 08 25.99416(W)   ADJUSTED

AI4510* NAD 83(2011) ELLIP HT-   -28.259 (meters)        (12/??/12)   ADJUSTED

AI4510* NAD 83(2011) EPOCH   -  2010.00

AI4510* NAVD 88 ORTHO HEIGHT -     4.30   (meters)      14.1   (feet) GPS OBS

AI4510  ______________________________________________________________________

AI4510  NAVD 88 orthometric height was determined with geoid model    GEOID03

AI4510  GEOID HEIGHT    -        -32.47  (meters)                     GEOID03

AI4510  GEOID HEIGHT    -        -32.52  (meters)                     GEOID12A

AI4510  NAD 83(2011) X  - -2,689,809.352 (meters)                     COMP

AI4510  NAD 83(2011) Y  - -4,281,189.079 (meters)                     COMP

AI4510  NAD 83(2011) Z  -  3,874,973.548 (meters)                     COMP

AI4510  LAPLACE CORR    -          2.63  (seconds)                    DEFLEC12A

AI4510  HORZ ORDER      -  SPECIAL (CORS)

AI4510  ELLP ORDER      -  SPECIAL (CORS)

AI4510

AI4510.Formal positional accuracy estimates are not available for this CORS

AI4510.because its coordinates were determined in part using modeled

AI4510.velocities. Approximate one-sigma accuracies for latitude, longitude,

AI4510.and ellipsoid height can be obtained from the short-term time series.

AI4510.Additional information regarding modeled velocities is available on

AI4510.the CORS Coordinates and Multi-Year CORS Solution FAQ web pages.

AI4510.The horizontal coordinates were established by GPS observations

AI4510.and adjusted by the National Geodetic Survey in December 2012.

AI4510

AI4510.NAD 83(2011) refers to NAD 83 coordinates where the reference

AI4510.frame has been affixed to the stable North American Tectonic Plate.

AI4510

AI4510.The horizontal coordinates are valid at the epoch date displayed above

AI4510.which is a decimal equivalence of Year/Month/Day.

AI4510

AI4510.The orthometric height was determined by GPS observations and a

AI4510.high-resolution geoid model using precise GPS observation and

AI4510.processing techniques.

AI4510

AI4510.The XYZ, and position/ellipsoidal ht. are equivalent.

AI4510

AI4510.The Laplace correction was computed from DEFLEC12A derived deflections.

AI4510

AI4510.The ellipsoidal height was determined by GPS observations

AI4510.and is referenced to NAD 83.

AI4510

AI4510. The following values were computed from the NAD 83(2011) position.

AI4510

AI4510;                    North         East     Units Scale Factor Converg.

AI4510;SPC CA 6     - 1,123,770.181 1,478,989.373   MT  1.00283790   -3 14 13.1

AI4510;SPC CA 6     - 3,686,902.67  4,852,317.63   sFT  1.00283790   -3 14 13.1

AI4510;UTM  10      - 4,167,623.257   575,812.323   MT  0.99967079   +0 31 30.1

AI4510

AI4510!             -  Elev Factor  x  Scale Factor =   Combined Factor

AI4510!SPC CA 6     -   1.00000443  x   1.00283790  =   1.00284235

AI4510!UTM  10      -   1.00000443  x   0.99967079  =   0.99967522

AI4510

AI4510                          SUPERSEDED SURVEY CONTROL

AI4510  NAVD 88 (04/03/01)    4.2    (m)  UNKNOWN model used   GPS OBS

AI4510  NAVD 88 (04/06/00)    4.3    (m)  GEOID99 model used   GPS OBS

AI4510

AI4510.Superseded values are not recommended for survey control.

AI4510

AI4510.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

AI4510.See file dsdata.txt to determine how the superseded data were derived.

AI4510

AI4510_U.S. NATIONAL GRID SPATIAL ADDRESS: 10SEG7581267623(NAD 83)

AI4510_MARKER: Z = SEE DESCRIPTION

AI4510_SETTING: 0 = UNSPECIFIED SETTING

AI4510_STAMPING: UNKNOWN

AI4510_MARK LOGO: NONE

AI4510_MAGNETIC: N = NO MAGNETIC MATERIAL

AI4510_STABILITY: A = MOST RELIABLE AND EXPECTED TO HOLD

AI4510+STABILITY: POSITION/ELEVATION WELL

AI4510_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

AI4510+SATELLITE: SATELLITE OBSERVATIONS - 1998

AI4510

AI4510  HISTORY     - Date     Condition        Report By

AI4510  HISTORY     - 1998     MONUMENTED       NGS

AI4510                          STATION DESCRIPTION

AI4510'DESCRIBED BY NATIONAL GEODETIC SURVEY 1998

AI4510'THIS MONUMENT IS ASSOCIATED WITH CORS SITE 'WINT'

AI4510'LATEST INFORMATION INCLUDING POSITIONS AND VELOCITIES

AI4510'ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

AI4510'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

AI4510'   ftp://cors.ngs.noaa.gov/cors/README.txt

AI4510'   ftp://cors.ngs.noaa.gov/cors/coord/coord_08

AI4510'   ftp://cors.ngs.noaa.gov/cors/station_log

AI4510'   http://geodesy.noaa.gov/CORS

______________________________________________________________________________